LAB VIVA-VOCE Questions & Answers

- What is crimping? Why we need crimping?

- Crimping is a process used to join wire to wire or wire to terminal.

- Crimp connectors are typically used to terminate stranded wire. The benefits of crimping over soldering and wire wrapping include: A well-engineered and well-executed crimp is designed to be gas-tight, which prevents oxygen and moisture from reaching the metals (which are often different metals) and causing corrosion.

- Which connector we use in crimping a LAN cable?

- Ethernet connector (8P8C) having 8 pin slots.

- What is the difference between a straight cable and a cross cable?

- Straight through cable connects a computer with a cable or DSL modem's LAN port. Crossover cable connects with a router's LAN port with switch/hub normal port. You should use straight-through cable when you want to connect two devices of different types.

- What is router? Why we use the router?

- A router is a device that connects two or more packet-switched networks or subnetworks.

- It serves two primary functions: managing traffic between these networks by forwarding data packets to their intended IP addresses, and allowing multiple devices to use the same Internet connection.

- Why we need gateway?

- A gateway is a node (router) in a computer network, a key stopping point for data on its way to or from other networks. Thanks to gateways, we are able to communicate and send data back and forth. The Internet wouldn't be any use to us without gateways (as well as a lot of other hardware and software).

- What are the various types of network devices?

- Types of network devices

- Hub.

- Switch.

- Router.

- Bridge.

- Gateway.

- Modem.

- Repeater.

- Access Point.

- What is subnet mask?

- A subnet mask is a 32-bit number created by setting host bits to all 0s and setting network bits to all 1s. In this way, the subnet mask separates the IP address into the network and host addresses. The “255” address is always assigned to a broadcast address, and the “0” address is always assigned to a network address.

- What is sub-netting?

- Subnetting is the process of dividing one network into smaller networks. Collectively, the smaller networks are referred to as subnetworks (or subnets), and the singular subdivision is a subnetwork (more commonly referred to as a subnet).

- What do you mean by Network?

- A network is a collection of computers, servers, mainframes, network devices, peripherals, or other devices connected to allow data sharing. An example of a network is the Internet, which connects millions of people all over the world.

- What do you mean by Node?

- A node is a point of intersection/connection within a data communication network. In an environment where all devices are accessible through the network, these devices are all considered nodes. The individual definition of each node depends on the type of network it refers to.

- What do you mean by Network Topology?

- A network topology is the physical and logical arrangement of nodes and connections in a network. Nodes usually include devices such as switches, routers and software with switch and router features. Network topologies are often represented as a graph.

- What is Routers?

- A router is a network device that can receive the data , analyse it and transmit it to other networks . It is used to connect a Local Area Network to the internet. Using a routing table, it ensures that data packets are travelling through the best suitable paths to their destinations.

- What is the OSI model? Explain the Different layers of the OSI model.

- In the OSI reference model, the communications between a computing system are split into seven different abstraction layers: Physical, Data Link, Network, Transport, Session, Presentation, and Application.

- What is ping?

- Ping is a network utility that refers to the signal sent out across the network to another computer, which then sends its own signal back. This signal, which is measured in milliseconds (ms), lets you know how long it takes for a packet of data to travel from your computer to a server on the internet and back.

- What is the fast Ethernet?

- Fast Ethernet is an extension of the 10-megabit Ethernet standard. It runs on twisted pair or optical fiber cable in a star wired bus topology, similar to the IEEE standard 802.3i called 10BASE-T, itself an evolution of 10BASE5 (802.3) and 10BASE2 (802.3a).

- What is IP configuration?

- The IP Configuration window configures the Internet Protocol parameters, allowing the device to receive and send IP packets. In its factory default configuration, the switch operates as a multiport learning bridge with network connectivity provided by the ports on the switch.

- What is PDU?

- a protocol data unit (PDU) is a single unit of information transmitted among peer entities of a computer network. It is composed of protocol-specific control information and user data.

- What is topology? Which network topology uses terminators?

- Network topology is the topological structure of a network and may be depicted physically or logically.

- Bus topology is a type of network topology in which all the devices are connected to a single cable which is called as the backbone of the network.It consists of a terminator at each end of the cable.

- Where do we use Straight-through UTP cable?

- UTP cables are commonly used as telephone wired and ethernet cables in telephone connections and LAN (Local Area Networks). It is used to transfer audio signals and electronic data. These are also used in horizontal and backbone cabling subsystems.

- What is ARP? Explain the use of ARP?

- Address Resolution Protocol (ARP) is a procedure for mapping a dynamic IP address to a permanent physical machine address in a local area network (LAN). The physical machine address is also known as a media access control (MAC) address.

- What is RARP?

- Reverse Address Resolution Protocol (RARP) is a protocol a physical machine in a local area network (LAN) can use to request its IP address.

- What is ARP table?

- The ARP table is a collection of recently learned MAC addresses and corresponding IP addresses. The next time the host must transmit a frame, it will search for the address in local memory and use the address found there instead of issuing another ARP request, if possible.

- What is ICMP? At which layer do ICMP works?

- The Internet Control Message Protocol (ICMP) is a protocol that devices within a network use to communicate problems with data transmission.

- ICMP is implemented as part of the IP layer. So ICMP processing can be viewed as occurring parallel to, or as part of, IP processing. Therefore, in the topic on TCP/IP-based layered network, ICMP is shown as a layer 3 protocol.

- What is DHCP? Which port is used by DHCP?

- DHCP is a network protocol to used to configure IP networks. A DHCP server listens to UDP port 67 and dynamically assigns IP addresses and other network parameters to DHCP clients. These clients will listen for responses on UDP port 68.

- What is 3 way handshaking process?

- Three-Way HandShake or a TCP 3-way handshake is a process which is used in a TCP/IP network to make a connection between the server and client. It is a three-step process that requires both the client and server to exchange synchronization and acknowledgment packets before the real data communication process starts.