CIC 307 COMPUTER NETWORKS

QUESTION BANK WITH SOLUTIONS

UNIT-I: Data Communications and Switching

- Explain the key components of a data communication system. What are their roles in data transmission?

Ans. The key components of a data communication system include sender, receiver, transmission medium, message, protocol, and interface. The sender encodes the message and transmits it over the transmission medium. The receiver decodes the message and sends it to the destination.

- Describe the OSI model. How does it differ from the TCP/IP model?

Ans. The OSI model consists of seven layers: Physical, Data Link, Network, Transport, Session, Presentation, and Application. The TCP/IP model has four layers: Network Interface, Internet, Transport, and Application. The key difference is in the number of layers and the specific functions assigned to each layer.

3. What are the different transmission modes in the physical layer of a communication system? Explain each.

Ans. The three transmission modes are simplex, half-duplex, and full-duplex. Simplex allows one-way communication, half-duplex allows two-way communication, but not simultaneously, and full-duplex allows two-way communication simultaneously.

4. Compare and contrast guided and unguided media in data communication.

Ans. Guided media include cables and wires that use physical paths for data transmission. Unguided media, such as wireless communication, use air as the transmission medium. Guided media offer better security and reliability, while unguided media offer mobility.

5. Explain error detection and correction codes used in data communication. Provide examples.

Ans Error detection codes identify errors in data, while error correction codes can correct errors. Examples include parity bits for error detection and Hamming codes for error correction.

6. What are the different types of switching in data communication? Explain circuit switching and packet switching.

Ans  Circuit switching establishes a dedicated path for the entire duration of a call. Packet switching breaks data into packets and sends them independently. Circuit switching is used in traditional telephone networks, while packet switching is used in modern internet communication.

7. Describe the message switching approach. What are its advantages and disadvantages?

Ans Message switching involves storing the entire message at intermediate switches before forwarding. Advantages include simplicity, but it is inefficient for large messages due to storage requirements.

UNIT-II: Data Link Layer and Medium Access

8. What are the design issues in the data link layer? Explain their significance.

Ans Design issues include framing, error control, flow control, and addressing. They are essential for ensuring reliable and efficient data transmission.

9.Explain the Stop-and-Wait ARQ protocol. How does it handle errors in data transmission?

Ans Stop-and-Wait ARQ is a simple protocol that sends one frame at a time and waits for an acknowledgment. If an acknowledgment is not received, it assumes an error and retransmits the frame.

10. Compare and contrast Go-Back-N ARQ and Selective Repeat ARQ.

Ans Both are automatic repeat request (ARQ) protocols, but Go-Back-N retransmits multiple frames upon detecting an error, while Selective Repeat retransmits only the damaged frames.

11. Explain the HDLC (High-Level Data Link Control) protocol and its key features.

Ans HDLC is a widely used data link layer protocol with features like frame structure, error detection, and flow control.

12 What is PPP (Point-to-Point Protocol)? How does it establish a point-to-point connection?

Ans PPP is a data link layer protocol used to establish a direct connection between two devices, typically over serial lines. It uses the Link Control Protocol (LCP) to establish and configure the connection.

13.Describe the channel allocation problem in the medium access sublayer. What solutions are used to address this problem?

Ans The channel allocation problem involves multiple devices sharing a communication medium. Solutions include controlled access, channelization, and multiple access protocols like CSMA/CD.

14. Explain IEEE standards 802.3 and 802.11 for LANs and WLANs. What are their key differences?

Ans IEEE 802.3 (Ethernet) is used for wired LANs, while IEEE 802.11 (Wi-Fi) is used for wireless LANs. They differ in terms of physical media, access methods, and data rates.

15. What are high-speed LANs? Provide examples of high-speed LAN technologies.

Ans High-speed LANs provide faster data transmission than traditional LANs. Examples include Gigabit Ethernet, 10 Gigabit Ethernet, and fiber-optic LANs.

16. Explain the operation of token ring and token bus LANs. How do they control access to the network?

Ans Token ring and token bus LANs use a token-passing protocol. Devices must possess the token to transmit data, ensuring controlled and fair access.

17. What is FDDI (Fiber Distributed Data Interface)? How does it differ from Ethernet?

Ans FDDI is a high-speed LAN technology that uses fiber optics for data transmission. It differs from Ethernet in terms of medium, topology, and redundancy.

18. Define the functions of network devices like repeaters, hubs, switches, and bridges.

Ans Repeaters extend the range of a network, hubs connect devices in a star topology, switches forward data based on MAC addresses, and bridges connect and filter traffic between network segments.

Numericals:

Example 1: Hamming Code for Error Detection

Suppose you have a 7-bit data word, and you want to add three parity bits to detect errors. Calculate the parity bits and create the Hamming code.

Solution:

- Number the data bits and parity bits starting from 1 (bit 1, 2, 3, 4, 5, 6, 7).

- Place the parity bits at positions that are powers of 2 (1, 2, 4). The other positions will be for data bits.

- Calculate the values for the parity bits:

  - Parity Bit 1 (P1) checks bits 1, 3, 5, 7.

  - Parity Bit 2 (P2) checks bits 2, 3, 6, 7.

  - Parity Bit 4 (P4) checks bits 4, 5, 6, 7.

Now, you set the values of the parity bits according to the parity (even or odd). If the total count of "1"s in the bits it checks is odd, the parity bit is set to 1. Otherwise, it's set to 0.

- P1 = (1 + 0 + 1 + 1) % 2 = 1 (odd, so set to 1)

- P2 = (0 + 1 + 0 + 1) % 2 = 0 (even, so set to 0)

- P4 = (1 + 1 + 0 + 1) % 2 = 1 (odd, so set to 1)

So, the Hamming code with error detection is 1011001.

Example 2: Error Correction with Hamming Code

Suppose you received a Hamming code 1011101 with a single-bit error during transmission. Can you identify and correct the error?

Solution:

- Write the received code and label the positions (bit 1, 2, 3, 4, 5, 6, 7).

  - Received: 1 0 1 1 1 0 1

- Calculate the parity bits for the received code.

  - P1 = (1 + 0 + 1 + 1) % 2 = 1 (matches the received bit).

  - P2 = (0 + 1 + 0 + 1) % 2 = 0 (matches the received bit).

  - P4 = (1 + 1 + 0 + 1) % 2 = 1 (matches the received bit).

- Since all the parity bits in the received code match, it indicates there is no error in the parity bits.

- The error must be in the data bit position that corresponds to the incorrect parity bit.

  - In this case, P1 is incorrect, so the error is at position 1.

- Correct the error by flipping bit 1.

  - Corrected code: 1011101

Example 3: Double Error Detection

Suppose you have a Hamming code with 4 data bits and 3 parity bits. You receive the code 1010010. Can you determine if there are any errors?

Solution:

- Write the received code and label the positions (bit 1, 2, 3, 4, 5, 6, 7).

  - Received: 1 0 1 0 0 1 0

- Calculate the parity bits for the received code.

  - P1 = (1 + 0 + 1 + 0) % 2 = 0 (matches the received bit).

  - P2 = (0 + 1 + 0 + 1) % 2 = 0 (matches the received bit).

  - P4 = (1 + 0 + 0 + 1) % 2 = 0 (matches the received bit).

- All the parity bits match, indicating no errors in the parity bits.

- Since the parity bits are correct, there are no single errors in the data bits.

- Therefore, if there are any errors, they must be double errors, and the code is not capable of detecting and correcting them.

Example 4: Odd Parity Bit Error Detection

Suppose you have a 7-bit data word with odd parity and a parity bit. The data word is 1011010, and the parity bit is used to check the odd parity. Calculate the parity bit and check if there is an error.

Solution:

- Count the number of "1"s in the data word (1011010).

  - Number of "1"s = 4

- Determine if the data word has odd parity (i.e., an odd number of "1"s).

  - The number of "1"s is 4, which is even.

  - To maintain odd parity, the parity bit should be set to 1.

- The full data word with the parity bit is 11011010.

- If any bit is flipped during transmission, you can detect the error using the parity check:

  - Received: 11111010 (error in the 2nd bit)

The received word has an even number of "1"s (5), which violates the odd parity rule. Therefore, you can detect the error.

Example 5: Even Parity Bit Error Detection

Suppose you have an 8-bit data word with even parity and a parity bit. The data word is 10011001, and the parity bit is used to check the even parity. Calculate the parity bit and check if there is an error.

Solution:

- Count the number of "1"s in the data word (10011001).

  - Number of "1"s = 4

- Determine if the data word has even parity (i.e., an even number of "1"s).

  - The number of "1"s is 4, which is already even.

  - To maintain even parity, the parity bit should be set to 0.

- The full data word with the parity bit is 010011001.

- If any bit is flipped during transmission, you can detect the error using the parity check:

  - Received: 010111001 (error in the 6th bit)

The received word has an odd number of "1"s (5), which violates the even parity rule. Therefore, you can detect the error.

Example 6: Error-Free Transmission

Suppose you have a 6-bit data word with even parity and a parity bit. The data word is 101010, and the parity bit is used to check the even parity. Calculate the parity bit and check if there is an error.

Solution:

- Count the number of "1"s in the data word (101010).

  - Number of "1"s = 3

- Determine if the data word has even parity (i.e., an even number of "1"s).

  - The number of "1"s is 3, which is odd.

  - To maintain even parity, the parity bit should be set to 1.

- The full data word with the parity bit is 1101010.

- If all bits are transmitted correctly, the received word matches the parity rules and has even parity. Therefore, there is no error.

In this case, the data word is transmitted without errors, and the parity check confirms that it has even parity.

Long Answers Questions

19.What is a computer network?

Ans. A computer network is a set of interconnected computers and other devices that can communicate and share resources with each other. These connections can be wired (e.g., Ethernet) or wireless (e.g., Wi-Fi). Computer networks enable data exchange, file sharing, internet access, and the ability to work collaboratively among connected devices.

20.Describe the modern uses of Computer Network.

Ans. Modern computer networks serve various purposes, including:

- Internet access for browsing, email, and online services.

- Sharing files and resources within a local network.

- Facilitating communication through email and messaging services.

- Enabling remote work and telecommuting.

- Supporting cloud computing and data storage.

- E-commerce and online transactions.

- Social media and content sharing.

- Internet of Things (IoT) connectivity.

- Video conferencing and streaming.

21.What is the principle of data communication? Describe briefly three modes of communication.

Ans. The principle of data communication is the exchange of data between two devices using a medium (e.g., cables, radio waves) with an agreed-upon protocol. The three modes of communication are:

- Simplex: Data flows in one direction only (e.g., TV broadcasting).

- Half-Duplex: Data flows in both directions, but not simultaneously (e.g., walkie-talkies).

- Full-Duplex: Data flows in both directions simultaneously (e.g., telephone conversations).

22. What is Intranet and Internet? Differentiate between LAN and WAN.

Ans. An Intranet is a private network within an organization that uses internet technologies for internal communication and information sharing.

The Internet is a global network that connects millions of public and private networks worldwide.

A LAN (Local Area Network) covers a small geographical area (like a home, office, or campus) and connects devices within that area.

A WAN (Wide Area Network) spans a larger geographical area, often connecting LANs or networks across cities or even countries.

23.Define physical and logical topology. Explain briefly different kinds of physical topology along with their advantages and disadvantages.

Ans.

- Physical Topology: This refers to the physical layout of devices and cables in a network. Common physical topologies include bus, star, ring, and mesh.

- Logical Topology: This defines how data flows in a network regardless of its physical layout. Common logical topologies include Ethernet, Token Ring, and IP-based networks.

- Brief advantages and disadvantages of physical topologies:

- Bus Topology: Simple, cost-effective, but a single cable failure can disrupt the entire network.

- Star Topology: Robust, easy to manage, but relies heavily on a central hub or switch.

- Ring Topology: Even data distribution, but a single cable or device failure can disrupt the network.

- Mesh Topology: High redundancy and fault tolerance but costly and complex to install.

24.What is the Client-Server model in Networking? Describe briefly 2-tier and 3-tier Client-Server architecture.

Ans. The Client-Server model is a network architecture where devices or processes (	clients) request services or resources from a central system (server). It's commonly used in distributed computing.

- 2-tier Client-Server architecture involves two main components: a client and a server. The client communicates directly with the server for tasks like data retrieval and processing. This model can be less scalable and flexible.

- 3-tier Client-Server architecture adds an intermediate layer (middleware or application server) between the client and the data server. This enhances scalability and separates business logic from data storage, making it more suitable for complex applications.

25.What is a wireless LAN? Mention the pros and cons of 802.11a and 802.11g wireless networking.

Ans.	A Wireless LAN (Local Area Network) is a network that uses wireless communication, such as Wi-Fi, to connect devices within a limited geographic area.

- 802.11a operates in the 5 GHz frequency range, offering higher data rates. Pros include less interference and more non-overlapping channels. Cons include shorter range and potential for signal degradation through obstacles.

- 802.11g operates in the 2.4 GHz frequency range, providing good range and compatibility. Pros include broader device support and longer range. Cons include potential interference and lower data rates compared to 802.11a.

26.What is the OSI reference model? Explain briefly all the 7 layers of the OSI reference model focusing on their functions, devices, and protocols working in each layer.

Ans.	The OSI (Open Systems Interconnection) reference model is a conceptual framework for understanding how network protocols work together. It consists of seven layers:

- Physical Layer: Manages the physical medium (cables, signals), responsible for bit transmission. Devices: Hubs. Protocols: Ethernet, USB.

- Data Link Layer: Handles framing and error detection. Devices: Switches, NICs. Protocols: Ethernet, Wi-Fi.

- Network Layer: Manages routing and logical addressing. Devices: Routers. Protocols: IP, ICMP.

- Transport Layer: Ensures end-to-end data delivery and flow control. Devices: Gateways. Protocols: TCP, UDP.

- Session Layer: Establishes, maintains, and terminates connections. Handles synchronization. Notable for NetBIOS.

- Presentation Layer: Translates, encrypts, or compresses data. Provides a common format. Handles data format issues.

- Application Layer: Interfaces between the user and the network. Contains application-specific protocols like HTTP, FTP, SMTP.

27. What are the advantages of OSI layers? Differentiate between TCP/IP layers and OSI layers.

  - Advantages of OSI layers:

      - Promotes interoperability and standardization.

      - Eases network troubleshooting and design.

      - Allows modular development of protocols.

  - TCP/IP is a more practical model used in the Internet. It combines some OSI layers:

      - Application Layer: Merges OSI's Presentation and Application layers.

      - Transport Layer: Corresponds to OSI's Transport Layer.

      - Internet Layer: Equivalent to OSI's Network Layer.

      - Link Layer: Combines OSI's Data Link and Physical layers.

28.What are guided and unguided transmission media? Differentiate between Hub and Switch.

- Guided transmission media are physical media that guide electromagnetic signals along a specific path, e.g., copper wires or optical fibers.

- Unguided transmission media allow signals to propagate freely, e.g., radio waves or microwaves.

- Hub: Operates at the Physical Layer, simply regenerates and broadcasts data to all connected devices, causing network congestion.

- Switch: Operates at the Data Link Layer, intelligently forwards data to the specific device based on its MAC address, reducing collisions and improving network efficiency.

29. Give two reasons for using layered protocols and explain the functions of various layers in the TCP/IP reference model. Two reasons for using layered protocols are:

- Modularity: Layered protocols allow for the separation of functionality, making it easier to modify or replace a single layer without affecting others.

- Interoperability: Layered protocols enable different vendors and systems to work together by adhering to standardized layers.

- The TCP/IP reference model consists of four layers:

- Network Interface Layer: Deals with hardware, addressing (MAC addresses), and frame transmission (e.g., Ethernet).

- Internet Layer: Manages routing, addressing (IP addresses), and packet forwarding (e.g., IP, ICMP).

- Transport Layer: Ensures end-to-end data delivery, flow control, and error checking (e.g., TCP, UDP).

- Application Layer: Contains various application-specific protocols (e.g., HTTP, FTP, SMTP) for user-level interaction.

30. Differentiate between broadband and baseband technology in communication.

Ans

- Broadband: Allows multiple signals to share a medium simultaneously. It's commonly used in cable TV and high-speed internet connections.

- Baseband: Uses the entire bandwidth of a medium for a single signal. It's common in digital communication, like Ethernet.

31.Show the comparison between Intranet, Extranet, Internet, and Piconet.

Ans

- Intranet: A private network within an organization for internal use.

- Extranet: An extension of an intranet that allows controlled external access (e.g., for partners or clients).

- Internet: A global public network accessible to everyone.

- Piconet: A small, ad-hoc wireless network often created by Bluetooth devices.

32.What is Framing? How does bit stuffing help to identify the beginning and end of a frame? Explain with suitable examples.

- Framing is the process of dividing a data stream into manageable frames for transmission.

- Bit stuffing is a technique used to mark the beginning and end of a frame. A special pattern is inserted into the data when certain conditions are met. For example, in HDLC (High-Level Data Link Control) framing, a frame starts with a flag pattern "01111110," and if this pattern appears in the data, it's stuffed with an extra "0" to distinguish it from the flag.

33. What is flow control? Briefly explain Stop and Wait and Sliding Window protocols.

Ans.

- Flow control is a mechanism used to manage the rate of data transmission between sender and receiver to prevent congestion or data loss.

- Stop and Wait: In this protocol, the sender sends one frame and waits for an acknowledgment from the receiver before sending the next frame. It's simple but not very efficient due to the wait time.

- Sliding Window: This protocol allows the sender to send multiple frames (a "window" of frames) before waiting for acknowledgments. It optimizes network efficiency by reducing idle time.

34. What is a parity bit? Suppose the information content of a packet is the bit pattern 1010101010101011, and an even parity scheme is being used. What would be the value of the checksum field in a single parity scheme?

Ans. A parity bit is an extra bit added to data to make the total number of "1" bits either even (even parity) or odd (odd parity).

In an even parity scheme, the total number of "1" bits in the data and the parity bit should be even. Given your pattern, which already has an even number of "1" bits, the parity bit would be "0" to maintain even parity.

35. What is CRC? Consider the 4-bit generator, G 1001, and suppose that the Data (Data that the sending node wants to send to the receiving node.) has the value 10101010. What is the value of R (additional bits)?

Ans. CRC (Cyclic Redundancy Check) is an error-checking method used to detect errors in data transmission. It involves dividing the data by a generator polynomial, and the remainder is sent along with the data.

In your example:

Data: 10101010

Generator: 1001

To compute R (the CRC remainder), perform a binary long division:

csharpCopy code

10101010000 (data with appended zeros)

/ 1001 1001101 -1001 1010 -1001 100 -100 00 ``` The remainder is 00.

36. Why is an ARP query sent within a broadcast frame? Why is an ARP response sent within a frame with a specific LAN address?

- ARP (Address Resolution Protocol) is used to map an IP address to a physical MAC address on a local network.

- An ARP query is broadcast to find the MAC address associated with a given IP address. It's sent as a broadcast because the sender doesn't know the target's MAC address.

- An ARP response is sent with a specific LAN address (MAC) because it's a unicast reply to the sender's query. Once the sender knows the MAC address, it can communicate directly.

37. Describe HDLC considering different fields in their frame format.

Ans HDLC (High-Level Data Link Control) is a data link layer protocol. Its frame format includes fields like:

Flag: Delimits the beginning and end of the frame (01111110).

Address: Specifies the destination address.

Control: Defines the frame type (e.g., command or response).

Information: Contains the data.

FCS (Frame Check Sequence): CRC-16 or CRC-32 used for error checking.

Flag: Marks the end of the frame.

38. Compare copper wire and fiber optics with respect to transmission speed, installation/maintenance, security, cost, effectiveness, reliability, and flexibility.

Ans Copper Wire:

- Slower transmission speed compared to fiber optics.

- Easier installation and maintenance.

- Less secure (can be tapped more easily).

- Lower cost (initially).

- Effective for shorter distances.

- Reliable but can be affected by interference.

- Less flexible (thicker cables).

Fiber Optics:

- Much higher transmission speed.

- Installation and maintenance are more complex.

- More secure (harder to tap).

- Higher initial cost.

- Effective for long distances.

- Highly reliable and immune to interference.

- More flexible (thinner cables).

39. Sketch the PPP frame format. Differentiate between CHAP and PAP.

- PPP (Point-to-Point Protocol) frame format typically includes fields like Flag, Address, Control, Protocol, Information, and FCS (Frame Check Sequence).

- PAP (Password Authentication Protocol) and CHAP (Challenge Handshake Authentication Protocol) are authentication protocols used in PPP:

  - PAP sends the username and password in plaintext, which is less secure.

  - CHAP uses a challenge-response mechanism where passwords are never sent in plaintext, enhancing security.

40. What are the perceived shortcomings of Serial Line Internet Protocol (SLIP), and how are these shortcomings removed by Point-to-Point Protocol (PPP)?

- SLIP (Serial Line Internet Protocol) is a simple protocol for serial communication but has several shortcomings, including a lack of error-checking, no support for multiple network layer protocols, and no authentication.

- PPP (Point-to-Point Protocol) addresses these shortcomings by introducing error-checking, support for multiple network layer protocols (like IP, IPX, and others), and authentication mechanisms like PAP and CHAP. PPP is a more comprehensive and versatile protocol for point-to-point communication.